RO-IR SRD-01

TECHNICAL REGULATION

for the radio interface

regarding non-specific proximity radio devices (short-range devices)

(previous coding RO-IR 01)

1. Basic Considerations

Directive 2014/53/EU of the European Parliament and of the Council of 16 April 2014 on the harmonization of the laws of the Member States relating to the making available on the market of radio equipment and repealing Directive 1999/5/EC was implemented in national legislation by Government Decision No. 740/2016 on making available on the market of radio equipment with subsequent amendments and completions.

This technical regulation contains the requirements for license-exempt use of radio spectrum by non-specific proximity radio device (short-range device) in the specified frequency bands and considers compliance, especially, with the provisions of Article 3 Paragraph 2 and Articles 6-8 of Directive 2014/53/EU.

Nothing in this technical regulation shall preclude the obligation for radio equipment placed on the market or made available on the market in Romania to comply with Directive 2014/53/EU.

The obligations arising from Directive (EU) 2015/1535 of the European Parliament and of the Council of 9 September 2015 laying down a procedure for the provision of information in the field of technical regulations and of rules on information society services are met in this regulation (OJ L 241, 17.9.2015, p. 1–15).

All the Romanian technical regulations for the radio interfaces notified under Directive (EU) 2015/1535 shall be published and shall be made available on National Authority for Management and Regulation in Communications (ANCOM) website at: http://www.ancom.ro/reglementari-interfete_2723 .

2. Radio Interface Specifications

Non-specific proximity radio device (short-range device)

| Frequency band | Annex |
| --- | --- |
| 442.2 – 450 kHz | RO-IR SRD-01-01 |
| 456.9 – 457.1 kHz | RO-IR SRD-01-02 |
| 13 553 – 13 567 kHz | RO-IR SRD-01-03 |
| 26 957 – 27 283 kHz | RO-IR SRD-01-04 |
| 26 990 – 27 000 kHz | RO-IR SRD-01-05 |
| 27 040 – 27 050 kHz | RO-IR SRD-01-06 |
| 27 090 – 27 100 kHz | RO-IR SRD-01-07 |
| 27 140 – 27 150 kHz | RO-IR SRD-01-08 |
| 27 190 – 27 200 kHz | RO-IR SRD-01-09 |
| 40.66 – 40.70 MHz | RO-IR SRD-01-10 |
| 87.5 – 108 MHz | RO-IR SRD-01-11 |
| 138.20 – 138.45 MHz | RO-IR SRD-01-12 |
| 169.4 – 169.475 MHz | RO-IR SRD-01-13 |
| 169.4 – 169.4875 MHz | RO-IR SRD-01-14 |
| 169.4875 – 169.5875 MHz | RO-IR SRD-01-15 |
| 169.5875 – 169.8125 MHz | RO-IR SRD-01-16 |
| 433.05 – 434.79 MHz | RO-IR SRD-01-17a; RO-IR SRD-01-17b |
| 434.04 – 434.79 MHz | RO-IR SRD-01-18 |
| 862 – 863 MHz | RO-IR SRD-01-19 |
| 863 – 865 MHz | RO-IR SRD-01-20 |
| 865 – 868 MHz | RO-IR SRD-01-21a; RO-IR SRD-01-21b |
| 868 – 868.6 MHz | RO-IR SRD-01-22 |
| 868.7 – 869.2 MHz | RO-IR SRD-01-23 |
| 869.4 – 869.65 MHz | RO-IR SRD-01-24 |
| 869.7 – 870 MHz | RO-IR SRD-01-25a; RO-IR SRD-01-25b |
| 874 - 874.4 MHz | RO-IR SRD-01-26 |
| 917.3 - 918.9 MHz | RO-IR SRD-01-27 |
| 916.1 - 919.4 MHz | RO-IR SRD-01-28 |
| 2 400 – 2 483.5 MHz | RO-IR SRD-01-29 |
| 5 725 – 5 875 MHz | RO-IR SRD-01-30 |
| 24 – 24.15 GHz | RO-IR SRD-01-31 |
| 24.15 – 24.25 GHz | RO-IR SRD-01-32 |
| 57 – 64 GHz | RO-IR SRD-01-33 |
| 61 – 61.5 GHz | RO-IR SRD-01-34 |
| 122 – 122.25 GHz 122.25 – 123 GHz | RO-IR SRD-01-35a RO-IR SRD-01-35b |
| 244 – 246 GHz | RO-IR SRD-01-36 |

For the purpose of this technical regulation, a proximity radio device (short-range device - SRD) means a radio device which provides either unidirectional or bidirectional communication, and which receives and/or transmits signals over a short distance and at low power.

The category of non-specific proximity radio devices (short-range device) covers all kinds of radio devices, regardless of the application or their purpose, which fulfil the technical conditions as specified for a given frequency band. Typical uses include telemetry, telecommand, alarms, data transmissions in general and other applications.

For the purpose of this technical regulation, a network access point in a data network is a fixed terrestrial short-range device that acts as a connection point for the other short-range devices in the data network to service platforms located outside of that data network. The term data network refers to several short-range devices, including the network access point, as network components and to the wireless connections between them.

For the purpose of this technical regulation, duty cycle means the ratio, expressed as a percentage, of Σ(Ton)/(Tobs) where Ton is the “on” time of a single transmitter device and Tobs is the observation period. Ton is measured in an observation frequency band (Fobs). Unless otherwise specified in this technical annex, Tobs is a continuous one hour period and Fobs is the applicable frequency band in this technical annex.

For the purpose of this technical regulation, non-interference and non-protected means that no harmful interference may be caused to any radio communication service and that no claim may be made for protection of these devices against interference originating from radio communication services.

The use of radio spectrum by proximity radio device (short-range device) is permitted on a non-interference and non-protected basis provided that such equipment meets the conditions set out in the Annex.

3. Document History:

| Edition | Changes |
| --- | --- |
| Edition 1/2014 | Notification number according to Directive 98/34/EC: 2014/597/RO. |
| Edition 2/2018 (06.08.2018) | Update according to Commission Implementing Decision (EU) 2017/1483 amending Decision 2006/771/EC on harmonization of the radio spectrum for use by short-range devices and repealing Decision 2006/804/EC: - RO-IR 01-01 ÷ RO-IR 01-31, except RO-IR 01-11 and RO-IR 01-26; - RO-IR 01-02 has been repealed (frequency band 6 765-6 795 kHz); - New technical specifications have been introduced: RO-IR 01-19b, RO-IR 01-30a, RO-IR 01-30b; Update of the legal framework according to Point 1 – „Basic considerations” and reference documents (line 13); Update according to the list of Class 1 radio equipment subclasses (version of January 2018) published according to Article 1 Paragraph (3) of Commission Decision 2000/299/EC (http://ec.europa.eu/docsroom/documents/26843): RO-IR 01-01; Formal changes according to TCAM-RSC model of November 2017. |
| Edition 3/2020 (23.12.2020) | Update of the legal framework according to Point 1 – „Basic considerations” and definitions to Point 2 - “Specifications for radio interface; Update according to Commission Implementing Decision (EU) 2019/1345 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices: - The following frequency bands have been introduced, the technical specifications being renumbered: 442.2-450 kHz (RO-IR 01-01); 862-863 MHz (RO-IR 01-18); - The rest of the technical specifications have been updated and renumbered Update according to Commission Implementing Decision (EU) 2018/1538 of 11 October 2018 on the harmonization of radio spectrum for use by short-range devices within the 874-876 and 915-921 MHz frequency: - The following frequency bands have been introduced: 874-874.4 MHz (RO-IR 01-25); 917.3-918.9 MHz (RO-IR 01-26); 917.4-919.4 MHz (RO-IR 01-27); Update according to the list of Class 1 radio equipment subclasses (version of January 2020) published according to Article 1 Paragraph (3) of Commission Decision 2000/299/EC (https://ec.europa.eu/docsroom/documents/40361). |
| Edition 4/2022 (11.10.2022) | Update according to Commission Implementing Decision (EU) 2022/172 amending Commission Implementing Decision (EU) 2018/1538 on the harmonization of radio spectrum for use by short-range devices within the 874-876 and 915-921 MHz frequency bands. Update according to Commission Implementing Decision (EU) 2022/180 amending Decision 2006/771/EC as regards the update of harmonised technical conditions in the area of radio spectrum use for short-range devices. - Update of reference documents (line 13) and definitions point 2. |
| Edition 5/2025 (22.12.2025) | Update in accordance with Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union: - The frequency band 87.5 – 108 MHz has been introduced (RO-IR SRD-01-11); - The rest of the technical specifications have been updated and renumbered. Update in accordance with Commission Implementing Decision (EU) 2025/650 amending Implementing Decision (EU) 2018/1538 as regards the update of harmonised technical conditions for short-range devices within the 874-876 and 915-921 MHz frequency bands. |

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-01 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) | This set of usage conditions is only available for person detection and collision avoidance devices. |
|  | 3 | Frequency band | 442.2 – 450.0 kHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | Channel spacing ≥150 Hz |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 7 dBμA/m at 10 meters |  |
|  | 8 | Channel occupation and access rules |  |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 330; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																	 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-02 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) | This set of usage conditions is only available for emergency detections of buried victims and valuable items devices. |
|  | 3 | Frequency band | 456.9 – 457.1 kHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 7 dBμA/m at 10 meters |  |
|  | 8 | Channel occupation and access rules |  |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | The application of Article 3(3)(g) of Directive 2014/53/EU, thereby ensuring access to emergency services in accordance with Commission Decision 2001/148/EC on the application of Article 3(3)(e) of Directive 1999/5/EC for avalanche warning light signals. |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 718; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																	 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-03 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 13 553 – 13 567 kHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 10 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 330; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR 																	 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-04 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 26 957 – 27 283 kHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 10 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; EN 300 330; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-05 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 26 990 – 27 000 kHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 100 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Duty cycle ≤0.1% | Model control devices (see RO-IR SRD-07-01) may operate without duty cycle restrictions. |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-06 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 27 040 – 27 050 kHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 100 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Duty cycle ≤0.1% | Model control devices (see RO-IR SRD-07-01) may operate without duty cycle restrictions. |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   																Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-07 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 27 090 – 27 100 kHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 100 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Duty cycle ≤0.1% | Model control devices (see RO-IR SRD-07-01) may operate without duty cycle restrictions. |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-08 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 27 140 – 27 150 kHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 100 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Duty cycle ≤0.1% | Model control devices (see RO-IR SRD-07-01) may operate without duty cycle restrictions. |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-09 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 27 190 – 27 200 kHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 100 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Duty cycle ≤0.1% | Model control devices (see RO-IR SRD-07-01) may operate without duty cycle restrictions. |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-10 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 40.66 – 40.70 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 10 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-11 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) | This set of usage conditions is only available for audio transmitters with analogue frequency modulation (FM). |
|  | 3 | Frequency band | 87.5 – 108 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | Bandwidth ≤ 200 kHz |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 50 nW effective radiated power (e.r.p.) | When audio signals are not present, the device must implement a transmission time out facility. Pilot tones that ensure continuity of transmission are not permitted |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 301 357; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-12 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 138.20 – 138.45 MHz |  |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel occupation and access rules | Duty cycle: ≤1% | When either duty cycle, Listen Before Talk (LBT) or equivalent technique applies then it shall not be user dependent/adjustable and shall be guaranteed by appropriate technical means. For LBT devices without Adaptive Frequency Agility (AFA), or equivalent techniques, the duty cycle limit applies. For any type of frequency agile device the duty cycle limit applies to the total transmission unless LBT or equivalent technique is used. |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; ERC/REC 70-03 |  |
|  | 14 | Notification number | 2014/597/RO |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-13 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 169.4 – 169.475 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 500 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Duty cycle: ≤1% For measuring devices (see RO-IR SRD-02-01) duty cycle ≤10%. | Duty cycle controls (hardware or software) related to the duty cycle requirements shall not be accessible to the user. The duty cycle shall not be capable of being disabled or altered and shall be implemented as an automatic feature in the equipment. |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ECC/DEC/(05)02; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-14 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 169.4 – 169.4875 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 10 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Duty cycle: ≤0.1% | Duty cycle controls (hardware or software) related to the duty cycle requirements shall not be accessible to the user. The duty cycle shall not be capable of being disabled or altered and shall be implemented as an automatic feature in the equipment. |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ECC/DEC/(05)02; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR 																	  Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-15 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 169.4875 – 169.5875 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 10 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Duty cycle: ≤0.001% Between 00:00h and 06:00h local time a duty cycle ≤ 0,1 % may be used. | Duty cycle controls (hardware or software) related to the duty cycle requirements shall not be accessible to the user. The duty cycle shall not be capable of being disabled or altered and shall be implemented as an automatic feature in the equipment. |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ECC/DEC/(05)02; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-16 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 169.5875 – 169.8125 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 10 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Duty cycle: ≤0.1% | Duty cycle controls (hardware or software) related to the duty cycle requirements shall not be accessible to the user. The duty cycle shall not be capable of being disabled or altered and shall be implemented as an automatic feature in the equipment. |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ECC/DEC/(05)02; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-17a | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 433.05 – 434.79 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 1 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules |  |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR 																	  Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-17b | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 433.05 – 434.79 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 10 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Duty cycle: ≤10% |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR 																	  Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-18 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 434.04 – 434.79 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 10 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Duty cycle: ≤100%, subject to bandwidth of ≤ 25 kHz. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-19 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 862 – 863 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | Bandwidth ≤ 350 kHz |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 25 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Duty cycle: ≤0.1% |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   																Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-20 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 863 – 865 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 25 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. Alternatively, a duty cycle ≤0.1% apply. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR 																	  Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-21a | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 865 – 868 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 25 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. Alternatively, a duty cycle ≤1% apply. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-21b | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) | This set of usage conditions is only available for data networks. |
|  | 3 | Frequency band | 865 – 868 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | Bandwidth: ≤ 200 kHz |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 500 mW effective radiated power (e.r.p.) Transmissions only permitted within the frequency bands: 865.6–865.8 MHz, 866.2–866.4 MHz, 866.8–867.0 MHz and 867.4–867.6 MHz. | Adaptive Power Control (APC) is required; alternatively, other mitigation techniques providing at least an equivalent level of spectral compatibility may be used. |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. Duty cycle: 10% for network access points Duty cycle 2.5% otherwise |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; EN 303 659; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-22 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 868 – 868.6 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 25 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. Alternatively, a duty cycle 1% applies. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR 																	  Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-23 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 868.7 – 869.2 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 25 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. Alternatively, a duty cycle 0.1% apply. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-24 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 869.40 – 869.65 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 500 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. Alternatively, a duty cycle 10% apply. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-25a | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 869.7 – 870 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 5 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules |  |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-25b | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 869.7 – 870 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 25 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. Alternatively, a duty cycle 1% apply. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-26 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

| Normative Part | 1 | Radiocommunication Service | Mobile |  |
| --- | --- | --- | --- | --- |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) | This set of usage conditions is only available for data networks All nomadic and mobile devices within the data network shall be controlled by a master network access point. |
|  | 3 | Frequency band | 874 – 874.4 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/650 amending Implementing Decision (EU) 2018/1538 as regards the update of harmonised technical conditions for short-range devices within the 874-876 and 915-921 MHz frequency bands) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | Bandwidth: ≤200 kHz |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 500 mW effective radiated power (e.r.p.) | Adaptive Power Control (APC) is required; alternatively, other mitigation techniques providing at least an equivalent level of spectral compatibility may be used. |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. Duty cycle: ≤10% for network access points. Duty cycle: 2.5% otherwise. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; EN 303 204; Commission Implementing Decision (EU) 2025/650 amending Implementing Decision (EU) 2018/1538 as regards the update of harmonised technical conditions for short-range devices within the 874-876 and 915-921 MHz frequency bands |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks |  |  |

F1- RTIR   															Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-27 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) | This set of usage conditions is only available for data networks All nomadic and mobile devices within the data network shall be controlled by a master network access point. |
|  | 3 | Frequency band | 917.3 – 918.9 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/650 amending Implementing Decision (EU) 2018/1538 as regards the update of harmonised technical conditions for short-range devices within the 874-876 and 915-921 MHz frequency bands) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | Bandwidth: ≤200 kHz |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 500 mW effective radiated power (e.r.p.) Transmissions only permitted within the bands of 917.3-917.7 MHz and 918.5- 918.9 MHz. | Adaptive Power Control (APC) is required; alternatively, other mitigation techniques providing at least an equivalent level of spectral compatibility may be used. |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. Duty cycle: ≤10% for network access points. Duty cycle: ≤2.5% otherwise. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; EN 303 659; Commission Implementing Decision (EU) 2025/650 amending Implementing Decision (EU) 2018/1538 as regards the update of harmonised technical conditions for short-range devices within the 874-876 and 915-921 MHz frequency bands; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks |  |  |

F1- RTIR 																  Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-28 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) | This set of usage conditions is only available for wideband SRD in data networks. All nomadic and mobile devices within the data network shall be controlled by a master network access point. |
|  | 3 | Frequency band | 916.1 – 919.4 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/650 amending Implementing Decision (EU) 2018/1538 as regards the update of harmonised technical conditions for short-range devices within the 874-876 and 915-921 MHz frequency bands). |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | Bandwidth: ≤600 kHz |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 25 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. Duty cycle: ≤1% |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; EN 303 659; Commission Implementing Decision (EU) 2025/650 amending Implementing Decision (EU) 2018/1538 as regards the update of harmonised technical conditions for short-range devices within the 874-876 and 915-921 MHz frequency bands; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks |  |  |

F1- RTIR  															 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-29 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 2 400 – 2 483.5 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 10 mW equivalent isotropic radiated power (e.i.r.p.) |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 440; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-30 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 5 725 – 5 875 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 25 mW equivalent isotropic radiated power (e.i.r.p.) |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 440; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-31 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 24 – 24.15 GHz |  |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 100 mW e.i.r.p. |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 440; ERC/REC 70-03 |  |
|  | 14 | Notification number | 2014/597/RO |  |
|  | 15 | Remarks | - |  |

F1- RTIR   																Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-32 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 24.15 – 24.25 GHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 100 mW equivalent isotropic radiated power (e.i.r.p.) |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 440; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-33 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 57 – 64 GHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 100 mW e.i.r.p. and a maximum transmit power of 10 dBm |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 305 550; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-34 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 61 – 61.5 GHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 100 mW equivalent isotropic radiated power (e.i.r.p.) |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 305 550; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-35a | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 122 – 122.25 GHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 10 dBm/250 MHz e.i.r.p. and – 48 dBm/MHz at 30° elevation |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 305 550; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR 																	  Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-35b | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 122.25 – 123 GHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 100 mW equivalent isotropic radiated power (e.i.r.p.) |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 305 550; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR 																	  Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific proximity radio devices (short-range devices) | RO-IR SRD-01-36 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | SRD / Non-specific proximity radio devices (short-range devices) |  |
|  | 3 | Frequency band | 244 – 246 GHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 100 mW equivalent isotropic radiated power (e.i.r.p.) |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 305 550; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR 																	  Edition:1; Revision:1