RO-IR SRD-02

TECHNICAL REGULATION

for the radio interface

concerning tracking, tracing and data acquisition devices

(Previous coding RO-IR 02)

1. Basic Considerations

Directive 2014/53/EU of the European Parliament and of the Council of 16 April 2014 on the harmonization of the laws of the Member States relating to the making available on the market of radio equipment and repealing Directive 1999/5/EC was implemented in national legislation by Government Decision No. 740/2016 on making available on the market of radio equipment with subsequent amendments and completions.

This technical regulation contains the requirements for license-exempt use of radio spectrum by short-range device designated for tracking, tracing and data acquisition devices in the specified frequency bands and considers compliance, especially, with the provisions of Article 3 Paragraph 2 and Articles 6-8 of Directive 2014/53/EU.

Nothing in this technical regulation shall preclude the obligation for radio equipment placed on the market or made available on the market in Romania to comply with Directive 2014/53/EU.

The obligations arising from Directive (EU) 2015/1535 of the European Parliament and of the Council of 9 September 2015 laying down a procedure for the provision of information in the field of technical regulations and of rules on information society services are met in this regulation (OJ L 241, 17.9.2015, p. 1–15).

All the Romanian technical regulations for the radio interfaces notified under Directive (EU) 2015/1535 shall be published and shall be made available on National Authority for Management and Regulation in Communications (ANCOM) web-site at: http://www.ancom.ro/reglementari-interfete_2723.

2. Radio Interface Specifications

Tracking, Tracing and Data Acquisition Devices

| Frequency band | Annex |
| --- | --- |
| 169.4 – 169.475 MHz | RO-IR SRD-02-01 |
| 430 – 440 MHz | RO-IR SRD-02-02 |
| 2 483.5 – 2 500 MHz | RO-IR SRD-02–03a; RO-IR SRD-02–03b |

For the purpose of this technical regulation, a proximity radio device (short-range device - SRD) means a radio device which provides either unidirectional or bidirectional communication, and which receives and/or transmits signals over a short distance and at low power.

For the purpose of this technical regulation, metering device means radio devices that are part of bidirectional radio communications systems which allow remote monitoring, measuring and transmission of data in smart grid infrastructures, such as electricity, gas and water.

For the purpose of this technical regulation, the medical data acquisition category covers the transmission of non-voice data to and from non-implantable medical devices for the purpose of monitoring, diagnosing and treating patients in healthcare facilities or patient's home as prescribed by duly authorised healthcare professionals.

For the purpose of this technical regulation, Medical Body Area Network Systems (MBANS), are used for medical data acquisition and are intended for low-power wireless networking of a plurality of body-worn sensors and/or actuators as well as of a hub device placed on/around the human body.

For the purpose of this technical regulation, duty cycle means the ratio, expressed as a percentage, of Σ(Ton)/(Tobs) where Ton is the “on” time of a single transmitter device and Tobs is the observation period. Ton is measured in an observation frequency band (Fobs). Unless otherwise specified in this technical annex, Tobs is a continuous one hour period and Fobs is the applicable frequency band in this technical annex.

For the purpose of this technical regulation, non-interference and non-protected means that no harmful interference may be caused to any radio communication service and that no claim may be made for protection of these devices against interference originating from radio communication services.

The use of radio spectrum by short-range devices is permitted on a non-interference and non-protected basis provided that such equipment meets the conditions set out in the Annex.

3. Document History:

| Edition | Changes |
| --- | --- |
| Edition 1/2014 | Notification number according to Directive 98/34/EC: 2014/598/RO. |
| Edition 2/2018 (06.08.2018) | Update according to Commission Implementing Decision (EU) 2017/1483 amending Decision 2006/771/EC on harmonization of the radio spectrum for use by short-range devices and repealing Decision 2006/804/EC: RO-IR 02-01 and have been introduced RO-IR 02-02a and RO-IR 02-02b (frequency band 2 483.5 – 2 500 MHz); Update of the legal framework according to Point 1 – „Basic considerations” and reference documents (line 13); Update according to the list of Class 1 radio equipment subclasses (version of January 2018) published according to Article 1 Paragraph (3) of Commission Decision 2000/299/EC (http://ec.europa.eu/docsroom/documents/26843): RO-IR 02-01; Formal changes according to TCAM-RSC model of November 2017. |
| Edition 3/2020 (23.12.2020) | Update of the legal framework according to Point 1 – „Basic considerations” and definitions to Point 2 - “Specifications for radio interface”; Update according to Commission Implementing Decision (EU) 2019/1345 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices. A new technical specification has been introduced: RO-IR 02-02 (430 – 440 MHz frequency band). Renumbering the technical specifications. Update according to the list of Class 1 radio equipment subclasses (version of January 2020) published according to Article 1 Paragraph (3) of Commission Decision 2000/299/EC (https://ec.europa.eu/docsroom/documents/40361). |
| Edition 4/2022 (11.10.2022) | Update according to Commission Implementing Decision (EU) 2022/180 amending Decision 2006/771/EC on the harmonization of radio spectrum for use by short-range devices. - Update of reference documents (line 13) and definitions point 2. |
| Edition 5/2025 (22.12.2025) | Update according to Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union: - Update of reference documents (line 13) and definitions point 2. |

| ROMANIA | Radio Interface Specification | SRD / Proximity radio devices (short-range devices) for tracking, tracing and data acquisition | RO-IR SRD-02-01 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio devices (short-range devices) / Metering devices Remote meter reading |  |
|  | 3 | Frequency band | 169.4 – 169.475 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | Max 50 kHz |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 500 mW effective radiated power (e.r.p.) |  |
|  | 8 | Channel occupation and access rules | For metering devices, duty cycle ≤ 10 % |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 220; ECC/DEC/(05)02; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Proximity radio devices (short-range devices) for tracking, tracing and data acquisition | RO-IR SRD-02-02 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio devices (short-range devices) / Devices used for medical data gathering | The set of usage conditions is only available for Ultra-Low Power Wireless Medical Capsule Endoscopy (ULP-WMCE) applications. Wireless medical capsule endoscopy is used for medical data acquisition designed for use in medical doctor-patient scenarios with the aim of acquiring images of human digestive tract. |
|  | 3 | Frequency band | 430 – 440 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) |  |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | – 50 dBm/100kHz e.r.p. power density but not exceeding a total power of – 40 dBm/10MHz (both limits are intended for measurement outside of the patient’s body) |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 303 520; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Proximity radio devices (short-range devices) for tracking, tracing and data acquisition | RO-IR SRD-02-03a | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio devices (short-range devices) / Devices used for the medical data gathering | The set of usage conditions is only available for Medical Body Area Network Systems (MBANS) for indoor use within healthcare facilities. |
|  | 3 | Frequency band | 2 483.5 – 2 500 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | Bandwidth ≤ 3 MHz |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 1 mW e.i.r.p. (equivalent isotropic radiated power) |  |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. In addition, a ≤10 % usage cycle applies. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 303 203; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Proximity radio devices (short-range devices) for tracking, tracing and data acquistion | RO-IR SRD-02-03b | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio devices (short-range devices) / Devices used for the medical data gathering | This set of conditions of use is only available for Medical Body Area Network Systems (MBANS) for indoor use within the patient’s home |
|  | 3 | Frequency band | 2 483.5 – 2 500 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | Bandwidth ≤ 3 MHz |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 10 mW e.i.r.p. |  |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. In addition, a ≤2 % usage cycle applies. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 303 203; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1