RO-IR SRD-04

TECHNICAL REGULATION

for the radio interface

concerning transport and traffic telematics devices (TTT)

(previous coding RO-IR 05)

1. Basic Considerations

Directive 2014/53/EU of the European Parliament and of the Council of 16 April 2014 on the harmonization of the laws of the Member States relating to the making available on the market of radio equipment and repealing Directive 1999/5/EC was implemented in national legislation by Government Decision No. 740/2016 on making available on the market of radio equipment with subsequent amendments and completions.

This technical regulation contains the requirements for license-exempt use of radio spectrum by transport and traffic telematics devices (TTT), including radar system installations to be used in vehicles, in the specified frequency bands and considers compliance, especially, with the provisions of Article 3 Paragraph 2 and Articles 6-8 of Directive 2014/53/EU.

Nothing in this technical regulation shall preclude the obligation for radio equipment placed on the market or made available on the market in Romania to comply with Directive 2014/53/EU.

The obligations arising from Directive (EU) 2015/1535 of the European Parliament and of the Council of 9 September 2015 laying down a procedure for the provision of information in the field of technical regulations and of rules on information society services are met in this regulation (OJ L 241, 17.9.2015, p. 1–15).

All the Romanian technical regulations for the radio interfaces notified under Directive (EU) 2015/1535 shall be published and shall be made available on National Authority for Management and Regulation in Communications (ANCOM) website at: http://www.ancom.ro/reglementari-interfete_2723.

2. Radio Interface Specifications

Transport and Traffic Telematics Devices (TTT)

| Frequency band | Annex |
| --- | --- |
| 984 – 7 484 kHz | RO-IR SRD-04-01 |
| 7 300 – 23 000 kHz | RO-IR SRD-04-02 |
| 5 795 – 5 815 MHz | RO-IR SRD-04-03a |
| 5 795 – 5 805 MHz | RO-IR SRD-04-03b |
| 5 855 – 5 865 MHz | RO-IR SRD-04-04 |
| 5 865 – 5 875 MHz | RO-IR SRD-04-05 |
| 24.05 – 24.075 GHz | RO-IR SRD-04-06 |
| 24.075 – 24.15 GHz | RO-IR SRD-04-07a; RO-IR SRD-04-07b |
| 24.15 – 24.25 GHz | RO-IR SRD-04-08 |
| 63.72 – 65.88 GHz | RO-IR SRD-04-09 |
| 76 – 77 GHz | RO-IR SRD-04-10a; RO-IR SRD-04-10b |

For the purpose of this technical regulation, a proximity radio device (short-range device - SRD) means a radio device which provides either unidirectional or bidirectional communication, and which receives and/or transmits signals over a short distance and at low power. Transport and traffic telematics devices are also part of short-range device category.

For the purpose of this technical regulation, the transport and traffic telematics devices category covers radio devices that are used in the fields of transport (road, rail, water or air, depending on the relevant technical restrictions), traffic management, navigation, mobility management and in intelligent transport systems (ITS). Typical applications include interfaces between different modes of transport, communication between vehicles (e.g. car to car), between vehicles and fixed locations (e.g. car to infrastructure) as well as communication from and to users.

For the purpose of this technical regulation, mean equivalent isotropically radiated power (‘e.i.r.p.’) means e.i.r.p. during the transmission burst which corresponds to the highest power, if power control is implemented.

For the purpose of this technical regulation, duty cycle means the ratio, expressed as a percentage, of Σ(Ton)/(Tobs) where Ton is the “on” time of a single transmitter device and Tobs is the observation period. Ton is measured in an observation frequency band (Fobs). Unless otherwise specified in this technical annex, Tobs is a continuous one hour period and Fobs is the applicable frequency band in this technical annex.

For the purpose of this technical regulation, eurobalise/euroloop means the transmission unit mounted on the track that uses the magnetic transponder technology. The main function of eurobalise/euroloop is to transmit and/or receive signals through the air. The eurobalise/euroloop is a single device mounted on the track, which communicates with the device mounted on a train passing over it.

For the purpose of this technical regulation, the magnetic transponder technology means the method that uses magnetic coupling between a transmitter and a receiver for conveying data and energy.

For the purpose of this technical regulation, smart tachographs are defined in Appendix 14 to  Commission Implementing Regulation (EU) 2016/799 (OJ L 139, 26.5.2016, p. 1) as  the application of the tachograph remote communication functionality, and applications of the weights and dimensions   are defined as the systems used to apply the weights and dimensions provisioned in Article 10d of Directive (EU) 2015/719 of the European Parliament and of the Council  (OJ L 115, 6.5.2015, p. 1).

For the purpose of this technical regulation, dwell time means in general, a time interval for which a certain frequency range is occupied.

For the purpose of this technical regulation, non-interference and non-protected means that no harmful interference may be caused to any radio communication service and that no claim may be made for protection of these devices against interference originating from radio communication services.

The use of radio spectrum by short-range devices is permitted on a non-interference and non-protected basis provided that such equipment meets the conditions set out in the Annexs.

3. Document History:

| Edition | Changes |
| --- | --- |
| Edition 1/2014 | Notification number according to Directive 98/34/EC: 2014/601/RO. |
| Edition 2/2018 (06.08.2018) | Update according to Commission Implementing Decision (EU) 2017/1483 amending Decision 2006/771/EC on harmonization of the radio spectrum for use by short-range devices and repealing Decision 2006/804/EC: - All technical specifications have been updated, except RO-IR 05-03b; - At RO-IR 05-03a, 5795 – 5805 MHz frequency band has been changed and has become 5795 – 5815 MHz, for road tolling infrastructure applications; - RO-IR 05-12b has been introduced for 76 – 77 GHz frequency band, concerning the conditions for the use of obstacle detection systems on gyroplanes (generic name given to aircraft with wings having a roundabout movement). Update of the legal framework according to Point 1 – „Basic considerations” and reference documents (line 13); Formal changes according to TCAM-RSC model of November 2017. |
| Edition 3/2020 (23.12.2020) | Update according to Commission Implementing Decision (EU) 2019/1345 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices: - Two new frequency bands have been introduced: 5855 – 5865 MHz (RO-IR 05-04) and 5865 – 5875 MHz (RO-IR 05-05); - Radio interfaces corresponding to the following frequency bands: 24.25 – 24.495 GHz, 24.25 – 24.5 GHz and 24.495 – 24.5 GHz have been removed; - 63 – 64 GHz frequency band corresponding to the former radio interface RO-IR 05-11 has been changed with the 63.72 – 65.88 GHz GHz frequency band (current RO-IR 05-09); - Extending the scope of the RO-IR 05–03a radio interface; - The Annexes have been renumbered; - Introduction/updating of some definitions and terms. Update according to the list of Class 1 radio equipment subclasses (version of January 2020) published according to Article 1 Paragraph (3) of Commission Decision 2000/299/EC (https://ec.europa.eu/docsroom/documents/40361); Update of the legal framework according to Point 1 – „Basic considerations” and reference documents (line 13). |
| Edition 4/2022 (11.10.2022) | Update according to Commission Implementing Decision (EU) 2022/180 amending Decision 2006/771/EC to update the technical requirements for radio spectrum use by short-range devices (SRDs) • Update of reference documents (line 13) and definitions point 2. |
| Edition 5/2025 (22.12.2025) | Update according to Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union. |

| ROMANIA | Radio Interface Specification | SRD / TTT | RO-IR SRD-04-01 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range) / Transport and traffic telematics devices (TTT) | This set of usage conditions is only available for transmissions (Eurobalises) in the presence of trains and using the 27 090-27 100 kHz band for telefeed, in accordance with the conditions established for the 26 957-272 83 kHz band (see RO-IR SRD-01-04). |
|  | 3 | Frequency band | 984 – 7 484 kHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 9 dBµA/m at 10 meters |  |
|  | 8 | Channel occupation and access rules | Duty cycle: ≤ 1 % |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 302 608; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / TTT | RO-IR SRD-04-02 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range) / Transport and traffic telematics devices (TTT) | This set of usage conditions is only available for Eurobalise transmissions in the presence of trains using the 27 090 -27 100 kHz band for telepowering pursuant to the conditions set for the 26957-27283 kHz band (see RO-IR SRD-01-04). |
|  | 3 | Frequency band | 7 300 – 23 000 kHz | Harmonized radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | – 7 dBµA/m at 10 meters |  |
|  | 8 | Channel occupation and access rules | Antenna restrictions shall be used | Antenna restrictions that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant restrictions are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these restrictions shall be ensured. |
|  | 9 | Authorization regime | Licence exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 302 609; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / TTT | RO-IR SRD-04-03a | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range) / Transport and traffic telematics devices (TTT) | This set of usage conditions applies only to road tolling applications and smart tachograph, weight and dimension applications. |
|  | 3 | Frequency band | 5 795 – 5 815 MHz | Harmonized radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 2 W equivalent isotropic radiated power (e.i.r.p.) |  |
|  | 8 | Channel occupation and access rules | Requirements on techniques to access spectrum and mitigate interference apply. | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. |
|  | 9 | Authorization regime | Licence exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 674-2-1; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / TTT | RO-IR SRD-04-03b | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range) / Transport and traffic telematics devices (TTT) |  |
|  | 3 | Frequency band | 5 795 – 5 805 MHz |  |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 8 W e.i.r.p. |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | Licence exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 674; ERC/REC 70-03 |  |
|  | 14 | Notification number | 2014/601/RO |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / TTT | RO-IR SRD-04-04 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range) / Transport and traffic telematics devices (TTT) | This set of usage conditions is only available for vehicle-to-vehicle, vehicle-to-infrastructure and infrastructure-to-vehicle systems. |
|  | 3 | Frequency band | 5 855 – 5 865 MHz | Harmonized radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 33 dBm e.i.r.p., 23 dBm/MHz e.i.r.p. density and a Transmit Power Control (TPC) able to reduce the total power from its maximum to 3 dBm e.i.r.p. |  |
|  | 8 | Channel occupation and access rules | Requirements on techniques to access radio spectrum and mitigate interference apply | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. |
|  | 9 | Authorization regime | Licence exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 302 571; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ECC/REC/(08)01 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / TTT | RO-IR SRD-04-05 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range) / Transport and traffic telematics devices (TTT) | This set of usage conditions is only available for vehicle-to-vehicle, vehicle-to-infrastructure and infrastructure-to-vehicle systems. |
|  | 3 | Frequency band | 5 865 – 5 875 MHz | Harmonized radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 33 dBm e.i.r.p., 23 dBm/MHz e.i.r.p. density and a Transmit Power Control (TPC) able to reduce the total power from its maximum to 3 dBm e.i.r.p. |  |
|  | 8 | Channel occupation and access rules | Requirements on techniques to access radio spectrum and mitigate interference apply | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. |
|  | 9 | Authorization regime | Licence exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 302 571; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ECC/REC/(08)01 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / TTT | RO-IR SRD-04-06 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range) / Transport and traffic telematics devices (TTT) |  |
|  | 3 | Frequency band | 24.05 – 24.075 GHz | Harmonized radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 100 mW e.i.r.p. |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | Licence exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 302 858; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / TTT | RO-IR SRD-04-07a | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range) / Transport and traffic telematics devices (TTT) | This set of usage conditions is only available for ground-based SAR systems. |
|  | 3 | Frequency band | 24.075 – 24.15 GHz | Harmonized radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | The range of frequency modulations provided in harmonized standards apply. |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 100 mW e.i.r.p. |  |
|  | 8 | Channel occupation and access rules | Requirements on techniques to access radio spectrum and mitigate interference apply | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. |
|  | 9 | Authorization regime | Licence exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 302 858; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / TTT | RO-IR SRD-04-07b | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range) / Transport and traffic telematics devices (TTT) |  |
|  | 3 | Frequency band | 24.075 – 24.15 GHz | Harmonized radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 0.1 mW e.i.r.p. |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | Licence exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 302 858; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / TTT | RO-IR SRD-04-08 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range) / Transport and traffic telematics devices (TTT) |  |
|  | 3 | Frequency band | 24.15 – 24.25 GHz | Harmonized radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 100 mW e.i.r.p. |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | Licence exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 302 858; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / TTT | RO-IR SRD-04-09 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range) / Transport and traffic telematics devices (TTT) | This set of usage conditions is only available for vehicle-to-vehicle, vehicle-to-infrastructure and infrastructure-to-vehicle systems. |
|  | 3 | Frequency band | 63.72 – 65.88 GHz | Harmonized radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 40 dBm e.i.r.p. |  |
|  | 8 | Channel occupation and access rules | TTT devices placed on the market before the 1 January 2020 are “grandfathered”, i.e. they are permitted to use the previous frequency range 63-64 GHz, and otherwise the same conditions apply. |  |
|  | 9 | Authorization regime | Licence exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 302 686; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ECC/DEC/(09)01; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / TTT | RO-IR SRD-04-10a | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range) / Transport and traffic telematics devices (TTT) | This set of usage conditions is only available for ground-based vehicle and infrastructure systems. |
|  | 3 | Frequency band | 76 – 77 GHz | Harmonized radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 55 dBm peak e.i.r.p., 50 dBm mean e.i.r.p. and 23.5 dBm for pulse radars |  |
|  | 8 | Channel occupation and access rules | Requirements on techniques to access spectrum and mitigate interference apply. Fixed transportation infrastructure radars have to be of a scanning nature in order to limit the illumination time and ensure a minimum silent time to achieve coexistence with automotive radar systems. | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. |
|  | 9 | Authorization regime | Licence exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 301 091; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / TTT | RO-IR SRD-04-10b | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range) / Transport and traffic telematics devices (TTT) | This set of usage conditions is only available for obstacle detection systems for rotorcraft use . Exclusion zones around radio astronomy sites shall apply. |
|  | 3 | Frequency band | 76 – 77 GHz | Harmonized radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 30 dBm peak e.i.r.p. and 3 dBm/MHz average e.i.r.p. density |  |
|  | 8 | Channel occupation and access rules | Duty cycle ≤ 56 % |  |
|  | 9 | Authorization regime | Licence exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 303 360; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonized technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonized technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ECC/DEC/(16)01; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1

Member States can specify exclusion zones or equivalent measures in which the obstacle detection application for rotorcraft use shall not be used for the protection of the radioastronomy service or other national use. Rotorcraft is defined as EASA CS-27 and CS-29 (resp. JAR-27 and JAR-29 for former certifications).;