RO-IR SRD-09

TECHNICAL REGULATION

for the radio interface

concerning radio-frequency identification devices (RFID)

(previous coding RO-IR 11)

1. Basic Considerations

Directive 2014/53/EU of the European Parliament and of the Council of 16 April 2014 on the harmonization of the laws of the Member States relating to the making available on the market of radio equipment and repealing Directive 1999/5/EC was implemented in national legislation by Government Decision No. 740/2016 on making available on the market of radio equipment with subsequent amendments and completions.

This technical regulation contains the requirements for license-exempt use of radio spectrum by radio-frequency identification devices (RFID) in the specified frequency bands and considers compliance, especially, with the provisions of Article 3 Paragraph 2 and Articles 6-8 of Directive 2014/53/EU.

Nothing in this technical regulation shall preclude the obligation for radio equipment placed on the market or made available on the market in Romania to comply with Directive 2014/53/EU.

The obligations arising from Directive (EU) 2015/1535 of the European Parliament and of the Council of 9 September 2015 laying down a procedure for the provision of information in the field of technical regulations and of rules on information society services are met in this regulation (OJ L 241, 17.9.2015, p. 1–15).

All the Romanian technical regulations for the radio interfaces notified under Directive (EU) 2015/1535 shall be published and shall be made available on National Authority for Management and Regulation in Communications (ANCOM) website at: http://www.ancom.ro/reglementari-interfete_2723.

2. Radio Interface Specifications

Radio-Frequency Identification Devices (RFID)

| Frequency band | Annex |
| --- | --- |
| 400 – 600 kHz | RO-IR SRD-09-01 |
| 13 553 – 13 567 kHz | RO-IR SRD-09-02 |
| 2 446 – 2 454 MHz | RO-IR SRD-09-03a; RO-IR SRD-09-03b |
| 865 – 868 MHz | RO-IR SRD-09-04 |
| 916.1 – 918.9 MHz | RO-IR SRD-09-05 |

For the purpose of this technical regulation, a proximity radio device (short-range device - SRD) means a radio device which provides either unidirectional or bidirectional communication, and which receives and/or transmits signals over a short distance and at low power.

The radio-frequency identification devices (RFID) category covers tag/interrogator based radio communications systems, consisting of (i) radio devices (tags) attached to animate or inanimate items; and (ii) transmitter/receiver units (interrogators) which activate the tags and receive data back. Typical applications include the tracking and identification of items, for instance for the purpose of electronic article surveillance (EAS), and collecting and transmitting data relating to the items to which tags are attached, which may be either battery-less, battery assisted or battery powered. The responses from a tag are validated by its interrogator and passed to its host system.

For the purpose of this technical regulation, non-interference and non-protected means that no harmful interference may be caused to any radio communication service and that no claim may be made for protection of these devices against interference originating from radio communication services.

The use of radio spectrum by short-range devices is permitted on a non-interference and non-protected basis provided that such equipment meets the conditions set out in the Annex.

3. Document History:

| Edition | Changes |
| --- | --- |
| Edition 1/2014 | Notification number according to Directive 98/34/EC: 2014/606/RO. |
| Edition 2/2018 (06.08.2018) | Update according to Commission Implementing Decision (EU) 2017/1483 amending Decision 2006/771/EC on harmonization of the radio spectrum for use by short-range devices and repealing Decision 2006/804/EC: - RO-IR 11-01, RO-IR 11-02, RO-IR 11-03a have been updated; - The new specification RO-IR 11-07 replacing RO-IR 11-04, RO-IR 11-05 and RO-IR 11-06 has been introduced. Update of the legal framework according to Point 1 – „Basic considerations” and reference documents (line 13); Formal changes according to TCAM-RSC pattern of November 2017. |
| Edition 3/2020 (23.12.2020) | Update according to Commission Implementing Decision (EU) 2019/1345 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices: Update of the legal framework according to Point 1 – „Basic considerations” and reference documents; Update according to Commission Implementing Decision (EU) 2018/1538 of 11 October 2018 on the harmonization of radio spectrum for use by short-range devices within the 874-876 and 915-921 MHz frequency bands: - 916.1-918.9 MHz (RO-IR 11-05) frequency band has been introduced; Renumbering of annexes; Update according to the list of Class 1 radio equipment subclasses (version of January 2020) published according to Article 1 Paragraph (3) of Commission Decision 2000/299/EC (https://ec.europa.eu/docsroom/documents/40361). |
| Edition 4/2022 (11.10.2022) | Update according to Commission Implementing Decision (EU) 2022/172 amending Commission Implementing Decision (EU) 2018/1538 on the harmonization of radio spectrum for use by short-range devices within the 874-876 and 915-921 MHz frequency bands. Update according to Commission Implementing Decision (EU) 2022/180 amending Decision 2006/771/EC to update the technical requirements for radio spectrum use by short-range devices (SRDs): - Update of reference documents (line 13) and definitions point 2. |
| Edition 5/2025 (22.12.2025) | Update according to Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union. Update according to Commission Implementing Decision (EU) 2025/650 amending Implementing Decision (EU) 2018/1538 as regards the update of harmonised technical conditions for short-range devices within the 874-876 and 915-921 MHz frequency bands. |

| ROMANIA | Radio Interface Specification | SRD / RFID | RO-IR SRD-09-01 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range device)/Radio-frequency identification devices (RFID) | In case of external antennas only loop coil antennas may be employed. |
|  | 3 | Frequency band | 400 – 600 kHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | Bandwidth ≥ 30 kHz |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | -8 dΒμΑ/m at 10 meters in any bandwidth of 10 kHz. Furthemore, the total field strength is −5 dBµA/m at 10 meters for systems operating in bandwidths larger than 10 kHz |  |
|  | 8 | Channel occupation and access rules | - |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 330; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  																 Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / RFID | RO-IR SRD-09-02 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range device)/Radio-frequency identification devices (RFID) |  |
|  | 3 | Frequency band | 13 553 – 13 567 kHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 60 dΒμΑ/m at 10 meters |  |
|  | 8 | Channel occupation and access rules | Transmission mask and antenna requirements for all combined frequency segments apply. | Antenna requirements that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant restrictions are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these restrictions shall be ensured. Transmission mask that provides an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant restrictions are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these restrictions shall be ensured. |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 330; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR 																	  Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / RFID | RO-IR SRD-09-03a | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range device)/Radio-frequency identification devices (RFID) |  |
|  | 3 | Frequency band | 2 446 – 2 454 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 500 mW equivalent isotropic radiated power (e.i.r.p.) |  |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 440; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   																Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / RFID | RO-IR SRD-09-03b | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range device)/Radio-frequency identification devices (RFID) |  |
|  | 3 | Frequency band | 2 446 – 2 454 MHz |  |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | > 500 mW up to 4 W e.i.r.p. | Power levels above 500 mW are restricted to be used inside the boundaries of a building and the duty cycle of all transmissions shall in this case be ≤ 15 % in any 200 ms period (30 ms on/170 ms off). |
|  | 8 | Channel occupation and access rules | Duty cycle: ≤ 15% Only FHSS techniques (Frequency Hopping Spread Spectrum) is used. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 300 440; ERC/REC 70-03 |  |
|  | 14 | Notification number | 2014/606/RO |  |
|  | 15 | Remarks | - |  |

F1- RTIR   																Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / RFID | RO-IR SRD-09-04 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range device)/Radio-frequency identification devices (RFID) | Operation only when necessary to perform the intended operation, i.e. when RFID tags are expected to be present. |
|  | 3 | Frequency band | 865 – 868 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | Bandwidth≤ 200 kHz |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 2 W effective radiated power (e.r.p.) Interrogator transmissions at 2 W e.r.p. only permitted within the channels centred at 865.7 MHz, 866.3 MHz, 866.9 MHz and 867.5 MHz. RFID tags respond at a very low power level (-20 dBm e.r.p.) in a frequency range around the RFID interrogator channels and shall comply with the essential requirements of Directive 2014/53/EU. | The RFID interrogators placed on the market before the repeal date of EC Decision 2006/804/EC are “grandfathered”, i.e. they are continuously permitted to be used in line with the provisions set out in EC Decision 2006/804/EC before the repeal date. |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 302 208; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   																Edition:1; Revision:1

| ROMANIA | Radio Interface Specification | SRD / Non-specific short-range devices | RO-IR SRD-09-05 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No. | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio device (short-range device)/Radio-frequency identification devices (RFID) |  |
|  | 3 | Frequency band | 916.1 – 918.9 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/650 amending Implementing Decision (EU) 2018/1538 as regards the update of harmonised technical conditions for short-range devices within the 874-876 and 915-921 MHz frequency bands). |
|  | 4 | Channeling (channel distribution) | - |  |
|  | 5 | Modulation/Occupied bandwidth | Bandwidth≤ 400 kHz |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 4 W e.r.p. Interrogator transmissions at 4 W e.r.p. only permitted the in the 916.3 MHz, 917.5 MHz and 918.7 MHz central frequencies. RFID tags respond at a very low power level (-10 dBm e.r.p.), in a frequency range around the RFID interrogator channels and shall comply with the essential requirements of Directive 2014/53/EU. |  |
|  | 8 | Channel occupation and access rules | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. |  |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 302 208; Commission Implementing Decision (EU) 2025/650 amending Implementing Decision (EU) 2018/1538 as regards the update of harmonised technical conditions for short-range devices within the 874-876 and 915-921 MHz frequency bands |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR  															 Edition:1; Revision:1