RO-IR SRD-12

TECHNICAL REGULATION

for the radio interface

concerning PMR446 devices

(previous coding RO-IR 15)

1. Basic Considerations

Directive 2014/53/EU of the European Parliament and of the Council of 16 April 2014 on the harmonization of the laws of the Member States relating to the making available on the market of radio equipment and repealing Directive 1999/5/EC was implemented in national legislation by Government Decision No. 740/2016 on making available on the market of radio equipment with subsequent amendments and completions.

This technical regulation contains the requirements for license-exempt use of radio spectrum by the devices that are intended for private mobile radio communications in the 446 – 446.2 MHz (PMR 446 devices) frequency band and considers compliance, especially, with the provisions of Article 3 Paragraph 2 and Articles 6-8 of Directive 2014/53/EU.

Nothing in this technical regulation shall preclude the obligation for radio equipment placed on the market or made available on the market in Romania to comply with Directive 2014/53/EU.

The obligations arising from Directive (EU) 2015/1535 of the European Parliament and of the Council of 9 September 2015 laying down a procedure for the provision of information in the field of technical regulations and of rules on information society services are met in this regulation (OJ L 241, 17.9.2015, p. 1–15).

All the Romanian technical regulations for the radio interfaces notified under Directive (EU) 2015/1535 shall be published and shall be made available on National Authority for Management and Regulation in Communications (ANCOM) website at: http://www.ancom.ro/reglementari-interfete_2723.

2. Radio Interface Specifications

PMR 446 Devices

| Frequency band | Annex |
| --- | --- |
| 446 – 446.2 MHz | RO-IR SRD-12-01 |

For the purpose of this technical regulation, a proximity radio device (short-range device - SRD) means a radio device which provides either unidirectional or bidirectional communication, and which receives and/or transmits signals over a short distance and at low power. Devices that are intended for private mobile radio communications (PMR 446) are also part of short-range device category.

For the purpose of this technical regulation, PMR446 devices covers hand portable equipment (without base station or repeater use) carried on a person or manually operated, which uses integral antennas only in order to maximise sharing and minimise interference. PMR 446 equipment operates in short-range peer-to-peer mode and shall not be used as a part of infrastructure network nor as a repeater.

For the purpose of this technical regulation, non-interference and non-protected means that no harmful interference may be caused to any radio communication service and that no claim may be made for protection of these devices against interference originating from radio communication services.

The use of radio spectrum by the PMR 446 devices is permitted on a non-interference and non-protected basis provided that such equipment meets the conditions set out in the Annex.

3. Document History:

| Edition | Changes |
| --- | --- |
| Edition 1/2011 | Notification number according to Directive 98/34/EC: 2011/99/RO. |
| Edition 2/2018 (18.10.2018) | Update according to Commission Implementing Decision (EU) 2017/1483 amending Decision 2006/771/EC on harmonization of the radio spectrum for use by short-range devices and repealing Decision 2006/804/EC: - Title change - Update of the radio interface specifications. Update of the legal framework according to Point 1 – „Basic considerations” and reference documents (line 13); Update according to the list of Class 1 radio equipment subclasses (version of January 2018) published according to Article 1 Paragraph (3) of Commission Decision 2000/299/EC (http ://ec.europa.eu/docsroom/documents/26843) ; Formal changes according to TCAM-RSC model of November 2017. |
| Edition 3/2020 (23.12.2020) | Update according to Commission Implementing Decision (EU) 2019/1345 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices: - Update of the radio interface specifications; - Introduction/update of some definitions and terms. Update of the legal framework according to Point 1 – „Basic considerations” and reference documents (line 13) as well; Update according to the list of Class 1 radio equipment subclasses (version of January 2020) published according to Article 1 Paragraph (3) of Commission Decision 2000/299/EC (https://ec.europa.eu/docsroom/documents/40361). |
| Edition 4 /2022 (11.10.2022) | Update according to Commission Implementing Decision (EU) 2022/180 amending Decision 2006/771/EC to update the technical requirements for radio spectrum use by short-range devices (SRDs): - Update of reference documents (line 13) and definitions point 2. |
| Edition 5/2025 (22.12.2025) | Update according to Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union: - Update of reference documents (line 13). |

| ROMANIA | Radio Interface Specification | SRD / PMR 446 Devices | RO-IR SRD-12-01 | Edition 5/2025 |
| --- | --- | --- | --- | --- |

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Mobile |  |
|  | 2 | Application | Proximity radio devices (short-range devices)/PMR 446 Private mobile radio communications devices |  |
|  | 3 | Frequency band | 446 – 446.2 MHz | Harmonised radio spectrum for use by short-range devices (Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union) |
|  | 4 | Channeling (channel distribution) | 6.25 kHz / 12.5 kHz | Carrier frequencies [MHz] for 12.5 kHz channel spacing: 446.006250; 446.018750; 446.031250; 446.043750; 446.056250; 446.068750; 446.081250; 446.093750; 446.106250; 446.118750; 446.131250; 446.143750; 446.156250; 446.168750; 446.181250; 446.193750 Carrier frequencies [MHz] for 6.25 kHz c channel spacing: 446.003125; 446.009375; 446.015625; 446.021875; 446.028125; 446.034375; 446.040625; 446.046875; 446.053125; 446.059375; 446.065625; 446.071875; 446.078125; 446.084375; 446.090625; 446.096875; 446.103125; 446.109375; 446.115625; 446.121875; 446.128125; 446.134375; 446.140625; 446.146875; 446.153125; 446.159375; 446.165625; 446.171875; 446.178125; 446.184375; 446.190625; 446.196875 |
|  | 5 | Modulation/Occupied bandwidth | - |  |
|  | 6 | Direction/Separation | - |  |
|  | 7 | Transmit power / Power density | 500 mW effective radiated power (e.r.p.) | The equipment shall use integral antennas only. |
|  | 8 | Channel occupation and access rules | Requirements on techniques to access spectrum and mitigate interference apply | Techniques to access spectrum and mitigate interference that provide an appropriate level of performance to comply with the essential requirements of Directive 2014/53/EU shall be used. If relevant techniques are described in harmonised standards or parts thereof the references of which have been published in the Official Journal of the European Union under Directive 2014/53/EU, performance at least equivalent to these techniques shall be ensured. |
|  | 9 | Authorization regime | License exemption |  |
|  | 10 | Additional essential requirements (According to Article 3 Paragraph 3 of 2014/53/EU Directive) | - |  |
|  | 11 | Assumptions on spectrum planning | - |  |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 303 405; Commission Implementing Decision (EU) 2025/105 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices and repealing Implementing Decision 2014/641/EU on harmonised technical conditions of radio spectrum use by wireless audio programme making and special events equipment in the Union; ECC/DEC/(15)05; ERC/REC 70-03 |  |
|  | 14 | Notification number | - |  |
|  | 15 | Remarks | - |  |

F1- RTIR   Edition:1; Revision:1